GuardianDash

Smart Driving.  Safer Lives.

Final Project — Embedded Systems Course

PRESENTED TO

Dr. Doaa Shoeib

Eng. Marwan  ·  Eng. Rehab

🛡  GuardianDash © 2026

# Team Members

GuardianDash was designed, built and shipped by a four-person team across hardware, firmware, mobile and backend layers.

| # | Name | Student ID |
| --- | --- | --- |
| 1 | Mohamed Abdallah Eldairouty | 221001719 |
| 2 | Rimas Emad | 221001067 |
| 3 | Judy Yehia | 221000495 |
| 4 | Moaz Ali | 221001970 |

# Project Overview

GuardianDash is a smart vehicle black-box that watches over the driver on every trip. A small STM32-based device inside the car continuously reads G-force from an MPU6050 accelerometer. The moment it detects a collision it shows STATUS: UNSAFE on its on-board LCD, streams the spike to a paired smartphone over Bluetooth, and automatically calls or messages the driver’s emergency contacts — all in seconds.

The system is end-to-end and standalone: the hardware lives sealed inside the vehicle on a USB power bank, the mobile app runs as a real native Android APK, and a small cloud backend persists user accounts and trip history. No laptop is required during a real drive.

# System Architecture

The product is split into three independent components. Each one can be developed, tested and deployed on its own, and they communicate through well-defined wireless interfaces.

🛰 Hardware — Vehicle_BlackBox • STM32F401 microcontroller running custom firmware in C • MPU6050 IMU over I²C, computing G = √(Ax² + Ay² + Az²) • 16×2 I²C LCD showing live G-force and SAFE / UNSAFE status • HM-10 BLE module streaming telemetry to the phone over Bluetooth • Powered by a USB power bank for full mobility

📱 Mobile App — GuardianDash (Android APK) • React Native + Expo + TypeScript codebase • Live dashboard mirroring the on-board LCD in real time • Full-screen crash alert with 30-second auto-call countdown • Bluetooth pairing flow, emergency contacts CRUD, trip history • Real GPS via expo-location, dark themed UI throughout

☁️ Cloud Backend — GuardianDash API • Node.js + Express REST API with JWT authentication • SQLite database storing users, contacts, trips and stats • Hosted on Render.com (free tier) over HTTPS • Auto-deploys from GitHub on every push

# Key Features

## Crash detection & emergency response

- Configurable G-force threshold (Low 2.0g  ·  Medium 1.5g  ·  High 1.2g)

- Full-screen red alert with 30-second cancel window

- Auto-call to the highest-priority emergency contact

- Parallel SMS draft with location and severity attached

- GPS coordinates included in the outgoing alert

## Live telemetry

- Real-time G-force readout mirroring the hardware LCD

- Accelerometer X / Y / Z displayed alongside total G

- Trip history with map polyline, distance, max and average speed

- Cloud sync — same account works across multiple devices

## Polished native UX

- Dark, modern UI built for clarity in any lighting

- Entrance animations and haptic feedback on every action

- Native Android APK — installs and runs without Expo Go

# Tools & Technologies

The stack was chosen for productivity, maintainability and cost — every layer is free for educational and prototype use.

| Layer | Tools / Components |
| --- | --- |
| Mobile App | React Native · Expo SDK 51 · TypeScript · Expo Router · Reanimated · Zustand |
| Maps | OpenStreetMap tiles · react-native-svg |
| Bluetooth | react-native-ble-plx (HM-10 BLE) |
| Backend | Node.js · Express · SQLite · JWT |
| Hosting | Render.com (free tier · auto-deploy from GitHub) |
| Hardware | STM32F401 · MPU6050 IMU · 16×2 I²C LCD · HM-10 BLE · USB power bank |
| Firmware | C · STM32 HAL · I²C1 @ 100 kHz · USART2 @ 9600 baud |
| Dev Tools | STM32CubeIDE · EAS Build · Git / GitHub |

# Closing

GuardianDash bridges three disciplines we explored throughout the Embedded Systems course: low-level sensor acquisition on the STM32, wireless data transport via BLE, and end-user-facing mobile and cloud development. The result is a fully working prototype that turns an accelerometer reading into a real-world emergency response in under a second.

🛡  GuardianDash — Smart Driving.  Safer Lives.